ADV bearer	Mesh node

GATT

Proxy node

Mesh node

Mobile

Mobile :  Supports Only GATT Bearer does not support ADV

Proxy node: A device in mesh NW which support both ADV and GATT bearer

Mesh node : supports only ADV bearer

The proxy in a Bluetooth Mesh network is a feature of certain nodes that allows devices that are not mesh-aware (i.e., they don't support the Bluetooth Mesh Profile) to communicate with the mesh network.

Proxy Server can forward mesh messages between the advertising bearer and the GATT bearer.

Proxy nodes can act as intermediaries, translating messages from the non-mesh devices to the mesh network and vice versa. This allows, for instance, a smartphone without mesh support to control mesh devices through a proxy node.

The Proxy Service is another set of GATT services and characteristics that enable the proxy functionality.

This service allows non-mesh devices (e.g., smartphones or tablets) to communicate with the mesh network by connecting to a proxy node.

The Proxy Service includes characteristics for sending and receiving mesh messages encapsulated in GATT notifications and indications.

  Provisioning Phase:

Provisioning of a device is done using the provisioning protocol that sends provisioning PDUs. The provisioning PDUs are transmitted to an unprovisioned device using a Generic Provisioning layer(ADV bearer) or Proxy protocol layer(GATT bearer).

PB-GATT is a provisioning bearer used to provision a device using Proxy PDUs to encapsulate Provisioning PDUs within the Mesh Provisioning Service

- A new device needs to join the mesh network.

- The provisioning process starts with the provisioner (which could be a smartphone or a dedicated provisioning device).

- The Provisioning Service (GATT-based) is used if the provisioning is done over a BLE connection.

- The unprovisioned device gets network information and becomes a part of the mesh network.

  Proxy Phase:

- Once devices are part of the mesh network, they communicate using mesh-specific protocols.

- A non-mesh device (e.g., a smartphone without native mesh support) needs to interact with the mesh network.

- The non-mesh device connects to a proxy node using the Proxy Service (GATT-based).

- The proxy node translates the GATT messages into mesh messages and relays them to the mesh network.

Controlling Devices via Proxy:

- Your smartphone wants to control the lights.

- The smartphone connects to a proxy node (perhaps another light or a dedicated proxy device) using the Proxy Service.

- The smartphone sends control commands (e.g., "turn on the light") to the proxy node.

- The proxy node translates these commands into mesh messages and forwards them to the appropriate mesh nodes (the lights).