Project: BATTERY SERVICE

Lowe Level Design Document

Table of Contents

1	Introduction	2

1.1	Purpose	3

1.2	Terminology	3

1.3	References	3

2	Scope	3

3	Task List and Effort Estimation	4

4	System Architecture	5

5	License Details	7

6	Development Environment	8

6.1      HW Detail	9

6.2	Development host	9

6.3	Build tools and IDE	9

7	Configurations Supported	9

7.1	Compile time configuration	10

7.2	Initialization configuration	10

7.3	Runtime configuration	10

8	Application API Library	10

8.1	Structures	11

8.1.1	battery_value,battery_value_measurment	11

8.2	Enums and Macros	11

8.2.1	BT_BAS_VAL_MAX (100)	11

8.2.2	BT_UUID_BATTERY_SERVICE_VAL  0x180F	12

8.2.3	BT_UUID_BATTERY_LEVEL_MEASUREMENT_VAL  0x2A19	13

8.2.4	STACKSIZE 1024	13

8.2.5         BT_UUID_BATTERY_SERVICE……………………………………………………………………………...13

8.2.6	         BT_UUID_BATTERY_LEVEL_MEASUREMENT	13

8.3	Callback function declarations	13

8.3.1	read_char()	14

8.3.2	battery_ccc_cfg_changed()	14

8.3.3	bt_bs_notify()	14

8.4	Function Declarations	15

8.4.1	battery_notify_thread()	15

9	Appendix	16

Revision History

| S. No | Date | Version | Change | Author | Reviewer | Approver |
| --- | --- | --- | --- | --- | --- | --- |
| 1. | 15/06/23 | 1.0 1.1 | Design doc for Battery service | Sowmya Shetty | Sachin chougale |  |

# Introduction

## Purpose

The purpose of this document is to provide a comprehensive understanding of the Battery Service (BAS). It explains how the BAS enables efficient battery management and enhances user experience by allowing the monitoring of battery-related information and it provide the information about the battery level.

Service and Characteristics

- The Battery Service is structured as a primary service in the Generic Attribute Profile (GATT) hierarchy It serves as the main container for battery-related information and characteristics in a BLE device.

- The BAS includes several characteristics that provide battery-related data and information.

- Battery Level represents the current battery level as a percentage. It allows the client to monitor the remaining battery capacity.

- Battery Power State indicates the power state of the battery, such as charging, discharging, or idle. It provides information on the current operational status of the battery.

- Battery critical status indicates that the device may no longer function as expected due to low energy or service required

- Estimated Service Date indicates the estimated service or replacement date of the battery represented by this instance of the Battery Service

## Terminology

| Term | Explanation |
| --- | --- |
| BAS | Battery Service which enables the monitoring of battery-related information in BLE devices. |
| GATT | Generic Attribute Profile, defines the structure and behavior of services and characteristics |
| UUID | Universally Unique Identifier |
| BT | Bluetooth |

## References

- Bluetooth SIG specification for Battery Service (BAS):

https://www.bluetooth.com/specifications/specs/battery-service/

2. Bluetooth Core Specification

https://www.bluetooth.com/specifications/specs/core-specification-5-3/

# Scope

This document provides a detailed description of the Battery Service (BAS) in Bluetooth Low Energy (BLE) devices. It covers the BAS structure, characteristics, and associated UUIDs.

It covers the essential battery-related information that can be accessed and monitored through the BAS.

# Task List and Effort Estimation

| S.No | Task | Sub Task | EFFORT |
| --- | --- | --- | --- |
| 1 | Design Document Server(On Off Cluster) Role - LLD | Identify the Macros, Structures Needed for implementing/storing the data Bulb cluster End point | 1.5Days |
|  |  | Identify functions, callbacks Needed for the handling functionalities Bulb Cluster : ON Off Cluster | 1.5Days |
| 2 | Development of Bulb and Temprature sensor | 1. Create on_off_cluster_.h and define required macros and declare functions with parameter which are to be implemented | 1.5Days |
|  |  | 2.Define function to initialise all the cluster attributes | 0.5Day |
|  |  | 3. Initialise LED's and Buttons as per requirement | 1Day |
|  |  | 4. Define function to handle Handling commands recived from client (device callback) | 0.5Days |
|  |  | 5. Function to handle network signals(zboss signal handler) | 1Day |
|  |  | 6. Define handler for identify notifications | 1Day |
|  |  | 7. Take care of initialising scenes and loading current settings from scene (Calling api from zigbee library) | 1Day |
| 3 | Unit Testing | Unit Testing of Bulb with existing switch | 0.5Days |
| 4 | Bug Fixing | If any | 1Day |

# System Architecture

Application

Battery Service Profile (BAS)

BLE protocol stack

Physical layer

Host Controller Interface

Logical Link Control and Adaptation protocol

SMP

ATT

GAP

GATT

BLE Protocol Stack

Link layer

The above diagram represents the battery service profile system architecture. Battery service enables efficient monitoring and control of battery-related information. In this system architecture, data flows between the BLE stack, the BAS profile, and the application layer.

The BLE stack consists of several layers, including the physical layer (PHY), link layer, and host layer. The stack manages the transmission and reception of BLE packets, handles connection establishment, and provides services for device discovery and data exchange.

The profile layer, specifically the Battery Service, defines the operations and characteristics related to battery service. The profile consists of services, characteristics, and descriptors.

The application layer provides a user interface that allows users to interact with the device's battery features. This can include displaying battery information. The application layer can request and retrieve the data related to Battery service.

Battery service profile contains the characteristics which are listed below:

Battery Level: This characteristics in the battery service is Mandatory. This characteristic is used to describe the level of the battery. When read, the Battery Level characteristic returns the current battery level.The value of the battery level is represented as a percentage from 0 to 100, where 0 percent represents a battery that is fully discharged, and 100 percent represents a battery that is fully charged

Battery Level descriptor: Characteristic Presentation Format descriptor

When a device has more than one instance of the Battery Service, each Battery Level characteristic shall include a Characteristic Presentation Format descriptor. Name Space field set to ”Bluetooth SIG” and the Description field set to a valid value from the GATT Namespace Descriptors. If the service that instantiates the Battery Level characteristic represents the battery status for the device, the Description field should be set to “main”.

Battery Level Status:

This characteristic used to describe the battery level and power state of the battery represented by this instance of the Battery Service. It provides a summary of battery status information when read. It will be indicated or notified if the battery level changes significantly, power state or service requirements change, upon reconnection with a bonded client, or when the Battery Level characteristic itself is being notified. It should not be indicated or notified for any other reason. The Power State field shall be included in the Battery Level Status characteristic. whether it is connected to an external power source, and whether it is charging or discharging.

Estimated Service Date:

The Estimated Service Date characteristic is used to provide an estimated date for the next battery service or replacement. Applications or devices can read this characteristic to inform users about the expected duration until the battery requires servicing or replacement. Notifications associated with this characteristic can help users stay informed about upcoming maintenance needs.

Battery Energy Status:

When the Battery Energy Status characteristic is read, it returns the details of the battery's energy status at that moment. It can provide information like the battery's capacity in kilowatt-hours or other quantitative measurements

Battery Time Status:

This characteristic may be used to describe the charge and discharge time status of the battery represented by this instance of the Battery Service.

Battery Health Status:

This characteristic may be used to describe several dynamic aspects of battery health for this instance of the Battery Service. The Battery Health Summary field may be included in the Battery Health Status characteristic. The field represents the aggregation of the overall health of the battery, where 100 percent represents new working condition and 0 percent represents the end of its expected lifetime when replacement is required.

Battery Health Information:

This characteristic may be used to describe static aspects of battery health for this instance of the Battery Service. The Battery Health Information characteristic gives detailed information about the battery's condition, such as capacity, voltage, and temperature, which helps users assess and manage the battery's health and performance.

Battery Information:

This characteristic may be used to describe static information about the battery represented by this instance of the Battery Service. Battery Information characteristic returns the physical characteristics of the battery.it should provide the information about the expiration date for the battery and manufacture date of the battery.

# License Details

NA For now!

# Development Environment

## HW Detail

Chip Name: nRF52840

Processing Speed: 64 MHz Cortex-M4 with FPU

Memory: 1 MB Flash, 256 KB RAM

Transceiver: 2.4 GHz Transceiver

Supported Tech: ANT, 802.15.4, Thread, Zigbee, Bluetooth Low Energy, Bluetooth mesh

Transmit power: +8 dBm TX Power

Interfaces supported: UART, SPI, TWI, PDM, I2S, QSPI

## Development host

Ubuntu Version: 22.04

Windows 11

## Build tools and IDE

Microsoft visual studio code

nRFConnect App

Seggar Jlink Tools

# Configurations Supported

## Compile time configuration

Kconfig file has multiple configurations compile time. This part will be added during development phase.

## Initialization configuration

## Runtime configuration

# Application API Library

## Structures

### battery_value, battery_value_measurment

struct bt_battery_value{ /** value handle.*/ uint16_t handle; /** Handle of the characteristic CCC descriptor. */ uint16_t ccc_handle; /** Read parameters. */ struct bt_gatt_read_params read_params; /** Read complete callback. */ bt_hrs_client_read_battery_value read_cb; /** GATT subscribe parameters for notification. */ struct bt_gatt_subscribe_params notify_params; /** Notification callback. */ bt_service_date_client_notify_cb notify_cb; }; struct bt_battery_value_measurement { /** Battery value Measurement . */ uint16_t battery_value; };

/* Maximum allowed value for battery level. */ #define BT_BAS_VAL_MAX (100) #define BT_UUID_BATTERY_SERVICE_VAL 0x180F /** * battery service */ #define BT_UUID_BATTERY_SERVICE BT_UUID_DECLARE_16(BT_UUID_CHAR1_BATTERY_SERVICE_VAL) #define BT_UUID_BATTERY_LEVEL_MEASUREMENT_VAL 0x2A19 /** * Battery level characteristic UUID value */ #define BT_UUID_BATTERY_LEVEL_MEASUREMENT \ BT_UUID_DECLARE_16(BT_UUID_BATTERY_LEVEL_MEASUREMENT_VAL) /* Stack size. */ #define STACKSIZE 1024

## Macro

## Callback function declarations

### read_char()

static ssize_t read_char(struct bt_conn *conn, const struct bt_gatt_attr *attr,

void *buf, uint16_t len, uint16_t offset)

Description: It will read the value from the buffer. The characteristic measurement value which are stored in the buffer, should be accessed by using this function. This function will returns the value which are stored in buffer.

Parameters: conn- pointer to the bluetooth connection object,

attr-pointer to the BT GATT attribute,

buf-buf pointer to the buffer where the value will be copied,

len-len length of the buffer,

offset- offset within the attribute value from which to start copying.

### battery_ccc_cfg_changed()

static void battery_ccc_cfg_changed(const struct bt_gatt_attr *attr, uint16_t value)

Description: This is used to handle changes in the Client Characteristic Configuration (CCC) descriptor of a Bluetooth GATT attribute. It will checks the Boolean value, if it is a true it will send a notification enabled message or else it will send the notification disabled message.

Parameters:
        	attr – it is a pointer to the BT GATT attribute.

Value -- new value of the CCC descriptor

### bt_bs_notify()

int bt_bs_notify (uint16_t meas_value)

Description: This function is to handle the notification about battery level. It will send the notification to the client about battery value measurement. An array will be declared to store the notification value and bt_gatt_notify function is called which will take first parameter to NULL indicating that the notification should be sent to all connected device, and the second parameter is bs_svc.attrs[4] which will indicate the notification sent to index 4 of the bs attribute structure ,&bs_char2 will points to the array which stored the notification vale and size of(bs_char2)indicating the size of the array.

Parameters:

meas_value---it is the measurement value for the notification.

## Function Declarations

### battery_notify_thread()

static void battery_notify_thread(void)

Description: This represents the thread that continuously sends the notification message. Here one structure variable is declared using that variable we are accessing the member of structure that hold the measurement value of battery level and assigning that value to the uint16_t battery_value variable,and calling the bt_bs_notify function is to send the notification.

# Appendix

About Capgemini

Capgemini is a global leader in partnering with companies to transform and manage their business by harnessing the power of technology. The Group is guided everyday by its purpose of unleashing human energy through technology for an inclusive and sustainable future. It is a responsible and diverse organization of over 325,000 team members more than fifty countries. With its strong 55-year heritage and deep industry expertise, Capgemini is trusted by its clients to address the entire breadth of their business needs, from strategy and design to operations, fueled by the fast evolving and innovative world of cloud, data, AI, connectivity, software, digital engineering and platforms. The Group reported in 2021 global revenues of €18 billion.

Get the Future You Want | www.capgemini.com

This document contains information that may be privileged 
or confidential and is the property of the Capgemini Group.

Choose an item. Copyright © 2022 Capgemini. All rights reserved.